# Definition

An LDR is a passive semiconductor device whose resistance decreases as light intensity increases.

# Working Principle (Photoconductivity)

An LDR works on the principle of Photoconductivity.

### Step 1 – No Light (Dark Condition)

- Very few electrons are free.

- Current cannot flow easily.

- Resistance becomes very high.

Result:

- High Resistance

- Low Current

### Step 2 – Light Falls on the LDR

When light falls on the LDR,

- Light particles (Photons) strike the semiconductor material.

- Electrons gain energy.

- More free electrons are created.

### Step 3 – Conductivity Increases

Because more free electrons are available,

- Current flows more easily.

- Resistance decreases rapidly.

### Step 4 – Arduino Reads the Voltage

The LDR and the 10kΩ resistor form a Voltage Divider.

Arduino reads the voltage at A0.

Depending on the light level,

- Dark → Lower Analog Voltage (according to this circuit configuration)

- Bright → Higher Analog Voltage

Arduino compares this value with the threshold and controls the LED.

LDR circuit tutorial with Arduino Uno

//CODE

// MADE BY SIR ANWAR SIRAJ DS & AI ROBOTICS

int ldrPin = A0;

int led = 13;

int threshold = 70;

void setup()

{

Serial.begin(9600);

pinMode(led, OUTPUT);

}

void loop()

{

int data = analogRead(ldrPin);

Serial.println("");

Serial.print("Light Sensor ");

Serial.print("Value = ");

Serial.print(data);

if(data <= threshold)

{

digitalWrite(led, HIGH);

}

else

{

digitalWrite(led, LOW);

}

}